AutoHV BiCMOS 180

Process Design Kit — Device Reference & User Guide

For schematic capture and netlisting in Qucs-S

38 devices · 5 process corners · process & mismatch statistics

# Contents

# 1. Overview

The AutoHV BiCMOS 180 PDK packages a 180 nm high-voltage BiCMOS process as a self-contained device library for Qucs-S. It is intended for schematic capture and netlist generation: you place devices on a schematic, wire them, set their parameters, and export a SPICE netlist for simulation in your target simulator. The library is written in ngspice-flavored SPICE.

The kit contains 38 devices spanning seven families: low-voltage core MOSFETs, high-voltage DMOS/LDMOS power transistors, bipolar transistors, diodes, Zener diodes, resistors, and capacitors. Every device supports five process corners selected by a single global parameter, and an orthogonal pair of switches enables die-to-die process variation and local device mismatch for statistical simulation.

Three global parameters control all 38 devices at once: case selects the process corner, PROC_ON enables process variation, and MM_ON enables mismatch. Their behavior is described in Section 3.

# 2. Library structure and installation

The PDK is delivered as two files that work as a pair:

| File | Contents | Scanned by the Qucs-S dock? |
| --- | --- | --- |
| autohv_bicmos180_case.lib | The 38 device .subckt wrappers. This is the file you reference from a netlist. | Yes — each .subckt becomes a device entry in the Libraries dock. |
| autohv_bicmos180_case_models.inc | Global control parameters, corner selectors, statistical parameters, and the 38 underlying .model cards. | No — the dock ignores .inc files, which keeps the model cards out of the device palette. |

The .lib file includes the .inc file on its first line (.include "autohv_bicmos180_case_models.inc"), so referencing the .lib is enough — but the two files must sit in the same folder, because the include uses a relative path.

The split is deliberate. The Qucs-S Libraries dock turns every top-level .model line it finds into a separate palette entry, which would flood the palette with dozens of junk items. Keeping the model cards in the .inc (which the dock does not scan) leaves the palette showing only the 38 real devices.

## 2.1 Installing into Qucs-S

Qucs-S builds the Libraries dock at startup by scanning *.lib files in up to three directories: the install library folder, your workspace user_lib folder, and the current project folder. To install the PDK, place the package in your workspace user library:

user_lib layout

<workspace>\user_lib\

autohv_bicmos180_case.lib            <- the library (scanned by the dock)

autohv_bicmos180_case_models.inc     <- models/params (sits beside the .lib)

autohv_bicmos180_case\               <- folder of symbols, named like the .lib

NMOS18.sym  NDMOS200.sym  ...     (one .sym per device)

The symbol folder must be named exactly autohv_bicmos180_case (matching the .lib base name) and sit beside the .lib. Qucs-S auto-resolves each device's symbol from <lib-folder>\<Device>.sym. After replacing library files, restart Qucs-S so the dock re-scans, and re-place any devices already on a schematic — the symbol is baked in at placement time.

Because the dock lists devices from the .lib, a stale copy of the .lib in any of the three scanned folders will keep showing old devices. If you ever see unexpected entries, click one and read the Library: path in the dock's info panel to find which file it came from.

# 3. Process corners and statistics

All corner and statistical behavior is driven by three global parameters defined at the top of the .inc file. Set them in your netlist (or leave them at their defaults) before the analysis statement; they apply to every device in the run.

## 3.1 Process corner — case

A single parameter, case, selects one process corner for the whole netlist. Each corner-dependent model value is a one-hot weighted sum keyed off case, so there is exactly one copy of each device rather than five.

| case | Corner | Meaning |
| --- | --- | --- |
| 0 | TT | Typical NMOS / typical PMOS (default) |
| 1 | FF | Fast NMOS / fast PMOS |
| 2 | SS | Slow NMOS / slow PMOS |
| 3 | FS | Fast NMOS / slow PMOS |
| 4 | SF | Slow NMOS / fast PMOS |

Set the corner with .param case=2 (for SS, say). To run a corner analysis, sweep case across runs (Section 7.2). Note that case is global: every device in a given run sits at the same corner, which is the standard way corner simulations are set up.

## 3.2 Statistics — PROC_ON and MM_ON

Two further switches, both defaulting to 0 (off), gate the statistical behavior. They are orthogonal — you can enable either, both, or neither:

| Parameter | Default | When set to 1 |
| --- | --- | --- |
| PROC_ON | 0 | Enables die-to-die process variation — a per-run random shift applied consistently to all devices of a type (the whole die is fast or slow together). |
| MM_ON | 0 | Enables local mismatch — an independent random offset per device instance, scaled by device size (larger devices mismatch less). |

For a nominal (deterministic) run, leave both at 0. For Monte Carlo, set the relevant switch(es) to 1 and re-run with fresh random draws (Section 7.3). Mismatch scales with size in the expected way: for the MOS and DMOS devices it follows a 1/√(W·M) law, and for the area-based devices (bipolars, diodes, Zeners) it scales as 1/√AREA. Resistors and capacitors scale as 1/√area, but their per-area mismatch coefficient is now layer-dependent rather than uniform: it grows with sheet resistance for the resistors (RPOLY_LO best, RNWELL worst) and is tighter for MIM than for metal capacitors, so two devices of equal area but different layers will show different spreads.

# 4. Device inventory

The 38 devices are grouped into seven families below. Port names are listed in pin order — that order is what matters when the netlist is generated (Section 5). Voltage classes for the MOS and DMOS devices are encoded in the device name (e.g. NMOS18 = 1.8 V, NDMOS200 = 200 V).

## 4.1 Core (low-voltage) MOSFETs

Four-terminal BSIM3 MOSFETs for low-voltage signal and logic design. Sized by drawn width and length, with an integer multiplier for parallel fingers.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| NMOS12 / PMOS12 | d g s b | W, L, M | 1.2 V core N- and P-channel FETs |
| NMOS18 / PMOS18 | d g s b | W, L, M | 1.8 V N- and P-channel FETs |
| NMOS33 / PMOS33 | d g s b | W, L, M | 3.3 V I/O-class N- and P-channel FETs |
| NMOS50 / PMOS50 | d g s b | W, L, M | 5.0 V N- and P-channel FETs |

Parameters: W drawn channel width, L drawn channel length, M integer device multiplier (parallel copies). Defaults are W=10u L=1u M=1. The fourth terminal b is the body/bulk and must be tied explicitly (usually to the source or the appropriate rail).

## 4.2 High-voltage DMOS / LDMOS

Three-terminal VDMOS power transistors for the high-voltage domain. These have no separate body terminal. They are sized by width; the 200 V N-channel device additionally exposes its drift length.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| NDMOS20 / 40 / 60 / 80 / 120 | d g s | W, M | N-channel LDMOS, 20-120 V classes |
| NDMOS200 | d g s | W, L, M | 200 V N-channel LDMOS — drift length L is adjustable |
| PDMOS20 / 40 / 60 / 80 | d g s | W, M | P-channel LDMOS, 20-80 V classes |
| DNMOS20 | d g s | W, M | 20 V drain-extended NMOS |

Width is the universal current knob: W scales drive current and 1/Rdson linearly via an internal device multiplier (total fingers = (W/W_REF)·M, with W_REF=10u). Defaults are W=10u M=1.

On NDMOS200 only, L sets the drift-region length (default L=8u). Increasing L raises the breakdown headroom of the real device and increases its on-resistance; the model captures the on-resistance rise (longer drift adds series drift resistance, scaled inversely with width). The recommended drift window is roughly 5 µm to 16 µm — the RESURF charge-balance range. The lower-voltage DMOS devices do not expose L; their length is fixed at the process minimum, as is typical for binned voltage classes. See Section 8 for calibration notes and the breakdown caveat.

## 4.3 Bipolar transistors

Three-terminal bipolar junction transistors, sized by emitter area.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| NPN_LV | c b e | AREA | Low-voltage vertical NPN |
| NPN_HV | c b e | AREA | High-voltage vertical NPN |
| PNP_HV | c b e | AREA | High-voltage PNP |
| PNP_LAT | c b e | AREA | Lateral PNP |

Parameter AREA (default 1) is the relative emitter-area multiplier, scaling current proportionally. Pin order is collector, base, emitter.

## 4.4 Diodes

Two-terminal junction diodes, sized by area.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| DIO_PN | a c | AREA | Standard PN-junction diode |
| DIO_FAST | a c | AREA | Fast-recovery diode |
| DIO_SCH | a c | AREA | Schottky diode (low forward drop) |

Pin order is anode, cathode. AREA (default 1) scales the junction area and hence current and capacitance.

## 4.5 Zener diodes

Two-terminal voltage-reference / clamp diodes. The breakdown voltage is encoded in the name.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| DZ_5V6 | a c | AREA | 5.6 V Zener |
| DZ_12 | a c | AREA | 12 V Zener |
| DZ_24 | a c | AREA | 24 V Zener |

Pin order is anode, cathode. Operate reverse-biased (cathode positive) for the clamp/reference function. AREA scales current capability.

## 4.6 Resistors

Two-terminal resistors across several process layers, sized by drawn length and width.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| RPOLY_HI | p n | L, W | High-sheet polysilicon resistor |
| RPOLY_LO | p n | L, W | Low-sheet polysilicon resistor |
| RNWELL | p n | L, W | N-well resistor (high value; high TC and large VCR) |
| RNPLUS | p n | L, W | N+ diffusion resistor |
| RPPLUS | p n | L, W | P+ diffusion resistor |

Resistance is set by the drawn geometry: a longer L raises resistance, a wider W lowers it. Defaults are L=100u W=10u. Pick the layer for the sheet resistance and matching/TC behavior you need.

The resistor models capture three layer-dependent behaviors. (1) Matching: the per-area mismatch coefficient grows with sheet resistance, so RPOLY_LO matches best, the diffusion layers and RPOLY_HI are progressively worse, and RNWELL is worst — reflecting the documented inverse relationship between resistance and matching coefficient in high-sheet poly. (2) Absolute (die-to-die) tolerance: low-sheet poly is tightest (±12% at 3σ), poly and diffusion are ±18%, and the lightly-doped n-well is widest (±30%). (3) Voltage coefficient (VCR): poly is nearly linear (~50–200 ppm/V), the diffusion layers are moderate (~1500–1800 ppm/V), and RNWELL is strongly nonlinear (~8000 ppm/V) from junction-depletion modulation — large enough to distort a filter or divider that carries signal swing, so the well resistor should be reserved for non-signal nodes. The n-well temperature coefficient was also raised to ~4000 ppm/°C to match published values for diffused/well resistors.

## 4.7 Capacitors

Two-terminal capacitors of several construction types, sized by plate length and width.

| Device | Ports | Parameters | Description |
| --- | --- | --- | --- |
| CMIM_STD | p n | L, W | Standard metal-insulator-metal capacitor |
| CMIM_HI | p n | L, W | High-density MIM capacitor |
| CMOM | p n | L, W | Metal-oxide-metal (interdigitated) capacitor |
| CFRINGE | p n | L, W | Lateral fringe capacitor |

Capacitance scales with plate area (L·W). Defaults are L=100u W=100u. Choose the type for the density, voltage linearity, and matching you need.

The capacitor models are also layer-aware. Matching follows the measured MIM-over-metal advantage: CMIM_STD matches best, CMIM_HI is close behind, and the metal caps (CMOM, CFRINGE) are several times looser. Each type carries a voltage coefficient (VCC): the MIM caps show a small bias dependence (~30–60 ppm/V, larger for the thinner-dielectric CMIM_HI), while the oxide-dielectric metal caps are nearly ideal (~3–5 ppm/V). VCC is usually negligible for filters, where the cap sees little DC swing, but it is present for completeness and for switched-cap or charge-redistribution use.

# 5. Symbols and port order

Each device has a schematic symbol whose pins map to the .subckt terminals positionally — by pin number, not by name. When Qucs-S generates a netlist it emits each device's nodes in pin-number order, so the order below is what determines correct connectivity.

| Family | Pin order (1 → N) | Symbol layout |
| --- | --- | --- |
| Core MOSFET (4-terminal) | 1=d 2=g 3=s 4=b | Drain top, gate left, source bottom, bulk right |
| HV DMOS (3-terminal) | 1=d 2=g 3=s | Drain top, gate left, source bottom (no bulk) |
| Bipolar | 1=c 2=b 3=e | Collector top, base left, emitter bottom |
| Diode / Zener | 1=a 2=c | Anode left, cathode right |
| Resistor | 1=p 2=n | Two-terminal zig-zag |
| Capacitor | 1=p 2=n | Two-terminal parallel-plate |

In a hand-written netlist this means the instance line lists nodes in exactly this order. For example X1 d g s b NMOS18 ... places drain, gate, source, bulk in pin order 1-4, and X2 d g s NDMOS200 ... places drain, gate, source in pin order 1-3.

# 6. Building circuits in Qucs-S

The workflow is schematic-first: assemble the circuit visually, then export a netlist.

- Open the Libraries dock and find the device under the autohv_bicmos180_case library. Drag it onto the schematic.

- Wire the pins. Watch the port layout from Section 5 — for the core FETs remember to connect the bulk (b) terminal.

- Double-click the device to open the SPICE library device dialog and set its parameters (W, L, M, or AREA depending on the family).

- Add your sources, ground, and a simulation block, then generate the netlist.

Each placed device references the library by name, so the generated netlist is an X (subcircuit) instance line plus the parameters you set. Keep the .lib and .inc together so the netlist resolves the models when you simulate.

After updating the library files, re-place devices that were already on a schematic so they pick up the current symbol and parameter set. This matters in particular for the DMOS devices: their sizing parameter changed from AREA to W / L / M, so an old instance carrying AREA=1 must be updated.

# 7. Running simulations

Reference the .lib from your deck and keep the .inc beside it. Set case / PROC_ON / MM_ON as needed. The examples below are ngspice-flavored; adapt the analysis and Monte Carlo syntax to your target simulator.

## 7.1 Nominal operating point

A typical-corner DC operating point with statistics off:

* Nominal OP - NMOS18 at TT

.include "autohv_bicmos180_case.lib"

.param case=0          ; TT corner

.param PROC_ON=0       ; process variation off

.param MM_ON=0         ; mismatch off

Vdd d 0 1.8

Vg  g 0 1.8

X1  d g 0 0 NMOS18 W=10u L=1u M=1   ; d g s b

.op

.end

## 7.2 Corner sweep

Step case over all five corners and read back the drain current drawn from the supply:

* Corner sweep - NMOS18, all five corners

.include "autohv_bicmos180_case.lib"

.param PROC_ON=0

.param MM_ON=0

Vdd d 0 1.8

Vg  g 0 1.8

X1  d g 0 0 NMOS18 W=10u L=1u

.control

foreach c 0 1 2 3 4

alterparam case=$c

reset

op

echo "case = $c"

print abs(vdd#branch)

end

.endc

.end

## 7.3 Statistical (Monte Carlo) run

Enable die-to-die process variation and/or local mismatch, then re-run with fresh random draws. In ngspice, reset inside a loop re-samples the AGAUSS draws; in a commercial simulator, switch the flags on and use its native Monte Carlo engine.

* Monte Carlo - mismatch + process on a current mirror pair

.include "autohv_bicmos180_case.lib"

.param case=0

.param PROC_ON=1       ; die-to-die process variation on

.param MM_ON=1         ; local mismatch on

Vdd dd 0 3.3

Iref dd in 10u

* diode-connected reference + mirror output device

X1 in in 0 0 NMOS33 W=20u L=1u

X2 out in 0 0 NMOS33 W=20u L=1u

Vout dd out 0

.control

let n = 200

let i = 0

dowhile i < n

reset

op

print abs(vout#branch)    ; mirror output current

let i = i + 1

end

.endc

.end

Because mismatch scales as 1/√(W·M), widening the mirror pair tightens the output-current distribution. Turn MM_ON off and leave PROC_ON on to see pure die-to-die spread, or vice-versa.

## 7.4 Sizing a high-voltage DMOS

Width scales current on every DMOS; on NDMOS200 the drift length L trades on-resistance against breakdown headroom. This deck compares two drift lengths at fixed width by measuring the on-state current into a small drain bias:

* NDMOS200 - on-resistance vs drift length (W fixed)

.include "autohv_bicmos180_case.lib"

.param case=0

Vg  g  0  5

Va  da 0  0.5

Vb  db 0  0.5

X_a da g 0 NDMOS200 W=100u L=8u    ; nominal drift

X_b db g 0 NDMOS200 W=100u L=16u   ; long drift -> higher Rdson

.control

op

print abs(va#branch) abs(vb#branch)   ; Id(8u) > Id(16u)

.endc

.end

Use M for integer arrays of fingers, e.g. W=40u L=10u M=4. For the lower-voltage DMOS, size by W (and M) only.

# 8. Notes, caveats, and calibration

A few modeling choices and limitations are worth knowing before you rely on the results quantitatively.

- DMOS drift-length values are placeholders. The NDMOS200 parameters L_REF=8u, L_MIN=5u, the ~5-16 µm window, and the drift-resistance coefficient are physically plausible defaults, not extracted from silicon. They live as named .param lines in the NDMOS200 subckt so you can calibrate them to your real process in one place.

- Breakdown is held at the model rating. Increasing L on the 200 V part raises modeled on-resistance but does not raise the simulated breakdown voltage — the breakdown stays at the model card's rating regardless of L. Capturing breakdown-versus-length would require a behavioral avalanche element or per-length model variants.

- Lower-voltage DMOS expose width only. Their channel and drift lengths are fixed at the process minimum, which is the normal behavior for binned voltage classes; only the top-of-range 200 V device gives a useful, characterized length window.

- Passive voltage/temperature coefficients are literature-anchored, not silicon-extracted. The resistor VCR values (poly ~50–200 ppm/V, diffusion ~1500–1800 ppm/V, n-well ~8000 ppm/V), the capacitor VCC values, the layer-dependent matching coefficients, and the raised n-well TCR (~4000 ppm/°C) are physically plausible defaults drawn from published data, not measured on this process. They live as named .param lines (VCR1/VCR2, VCC1/VCC2, RSH0/CJ0, and the per-layer mismatch and P_DRSH coefficients) inside each device subckt so you can recalibrate them in one place against real silicon.

- Resistor VCR uses the two-terminal voltage as a proxy for the true junction bias. The well and diffusion resistors expose only p and n terminals (no separate well/substrate tap), so their voltage coefficient is keyed to the magnitude of the terminal voltage rather than the actual depletion bias against the isolation. This captures the first-order nonlinearity but is not a substitute for a proper three-terminal well-resistor model if you need accurate large-signal behavior.

- Voltage coefficients are implemented behaviorally and preserve the existing models. VCR is a node-voltage-controlled source in series with the model-card resistor, so the resistor’s temperature, corner, mismatch, and flicker-noise behavior are untouched; VCC is a behavioral capacitor in parallel with the model-card capacitor. Both use node voltages only (for portability across ngspice and SmartSpice) and vanish at zero bias, so nominal values are unchanged. Two decks — autohv_passive_validation.cir and autohv_mismatch_mc.cir — exercise nominal value, TCR, VCR, VCC, corner spread, and layer-dependent mismatch.

- Parameter rename. DMOS devices now size by W / L / M rather than the former AREA. Re-place or update any DMOS instances that still carry AREA. Bipolars, diodes, and Zeners legitimately keep AREA.

- One corner per run. case is global, so a single simulation evaluates one corner. Sweep it across runs for corner analysis.

- Use plain .include. The library is a flat set with no .lib / .endl sections, so reference it with .include, not the sectioned .lib "file" section directive.

AutoHV BiCMOS 180 PDK — Device Reference & User Guide